# WORK CREW SAFETY GUIDELINE

A Safe Attitude

- Think Before you work….

- Inspect equipment before use.

- If you are uncertain about how to do a task or how to operate a power tool, ask your crew leader

- Know where water and the first aid kit are located.

- Report all injuries to the crew leader

- If something looks unsafe, tell you’re your crew leader

- Don’t lift beyond your strength. Bend your knees.

- Be aware of your surroundings and of the people working near you.

Proper Clothing

- Remove rings and jewelry.

- Secure loose clothing. Button or roll up sleeves. Zip jackets. Loose clothing is especially dangerous around power tools.

- Wear boots or thick-soled shoes. No Sandals!

A Clean Work Site

- Remove or bend back nails in discarded lumber immediately!

- Don’t leave tools or materials where others can trip over them.

- Use trashcans.

Tools

- Ask crew leader for instructions before using a tool if not thoroughly familiar with its use.

- No one under 18 may use power tools.

- Don’t place tools where they can fall on someone or trip someone.

- Don’t use power tools with defective switches, cords, guards, or plugs. Report defective tools to the crew leader.

- Don’t use a power tool if you’re standing on wet or damp ground.

- Use both hands if needed to operate a tool safely; if not, keep the other hand clear.

- Keep electric cords out of the way of the saw blade.

- Support material properly before cutting or drilling.

- Ask for help if a tool jams.

- Never lower or carry a power tool by its cord.

## OVER

Protective Gear

- Wear safety glasses when nailing or using power tools or solvents.

- Wear a hard hat when work is going overhead.

- Use ear protection when using power tools for prolonged periods.

- Use dust masks when working in dusty conditions.

Ladders

- No one under 18 may use a ladder.

- Inspect a ladder before using it. If it looks unsafe, DON’T USE IT!

- Only one person on a ladder at one time.

- Set ladder on solid ground. To level, dig a hole until it is stable.

- Use a ladder that will reach the work. An extension ladder should reach 3 feet above the work level.

- With toes touching ladder feel palms should rest on rung at shoulder eight to achieve proper extension ladder angle.

- Never stand on top rungs of a ladder.

- Don’t overreach.

- Keep ladders clear of power lines.

- Don’t leave loose objects on a ladder.

- Keep your tools in your tool belt at all times.

Roofing

- No one under 18 may work on the roof.

- Use toe boards or roof jacks.

- Keep rooftop swept free of sawdust.

- Keep rooftop swept free of sawdust and debris.

Scaffolding

- No one under 18 may work on scaffolding.

- Consult crew leader for proper scaffolding erection and bracing.

- Use extra caution when on scaffolding. Especially near ends. Watch your footing!

- Don’t leave loose objects on scaffolding.

- Keep your tools in your tool belt at all times.

Drywall

- Stack drywall material so that it is stable and secure.

- Never pull the stack away form the wall as shifting stacks have broken many bones. Drywall is very heavy!

Poisons and toxins

- The toxic substances, which can most often be found on a work site solvents and some cleaners. Special care must be taken when you come in contact with any of these substances, or any unfamiliar substance.

- Wear safety glasses and gloves when working with these substances.

- If you don’t know what a substance is or whether it is toxic, ask your crew leader.